### 0. Git repository setup (do this first)

- Create a repo on GitHub/GitLab (e.g., aquarium-auto-cleaner) and clone it locally

- Add a README.md with the project title, aim, and team members

- Add a .gitignore covering IDE files, build output, and secrets files (secrets.h, .env, config.json)

- Add a LICENSE (MIT is the usual choice for a college microproject)

- Create the folder structure:

### aquarium-auto-cleaner/

### ├── firmware/          # Arduino sketches

### │   ├── main/

### │   └── tests/         # single-sensor test sketches

### ├── bridge/            # ESP / serial-forwarding code (if used)

### ├── backend/           # server or database config

### ├── dashboard/         # web dashboard

### ├── docs/              # circuit diagram, flowchart, threshold table

### ├── .gitignore

### └── README.md

- Add secrets.example.h (with placeholder Wi-Fi/API values) and commit that instead of the real file

- Create a develop branch for work in progress, with main kept for working versions only

- Agree on a branch naming and commit message convention (e.g., feature/dashboard, feat: add turbidity calibration)

- Give each team member access and decide who owns which folder to avoid merge conflicts

### 1. Define the requirements

- List the sensors: turbidity, gas (e.g., MQ-135 or MQ-2), temperature (DS18B20 or DHT11), water level sensor

- Decide the threshold for each parameter (turbidity in NTU or raw ADC, gas in ppm or raw value, temperature in °C)

- Define the trigger rule: any 2 of the 3 parameters (turbidity, gas, temperature) crossing their thresholds sends the permission request

- Define the cleaning logic: pump runs until the water level sensor reads the target level, with a maximum run time as a safety cutoff

- Decide the dashboard refresh interval: 5 minutes

### 2. Hardware setup

- Wire the turbidity sensor to an analog pin (A0)

- Wire the gas sensor to an analog pin (A1)

- Wire the temperature sensor to a digital pin, with a 4.7 kΩ pull-up if using the DS18B20

- Wire the water level sensor to an analog pin (A2)

- Connect the pump through a relay module or a MOSFET/transistor with a flyback diode. Never drive the pump directly from an Arduino pin.

- Use a separate power supply for the pump, with a common ground with the Arduino

- Keep all electronics away from water; waterproof the probes and cable joints

- Note that the MQ gas sensors need a warm-up period (a few minutes, ideally longer on first use)

### 3. Test each sensor individually

- Read each sensor over Serial and confirm the values are sensible

- Calibrate the turbidity sensor (clean water vs. dirty water readings)

- Calibrate the gas sensor in clean air

- Check the water level sensor against known levels

- Test the relay and pump on their own

- Record the readings and finalize the threshold values

### 4. Arduino firmware

- Read all sensors at a fixed interval

- Average several samples per reading to reduce noise

- Compare the readings with the thresholds

- Implement a state machine: MONITORING → ALERT (awaiting permission) → CLEANING → DONE

- Add a cooldown so the same alert is not repeated every cycle

- Add safety logic: maximum pump run time, and stop if the water level sensor reads a fault or an unexpected value

- Send data in a simple format (e.g., JSON or CSV) over Serial

### 5. Connectivity (choose one)

The Uno has no built-in Wi-Fi, so pick a route:

- Option A: ESP8266/ESP32 as a Wi-Fi bridge, with the Uno sending data over serial (use a level shifter or voltage divider, since the Uno is 5 V and the ESP is 3.3 V)

- Option B: USB to a laptop or Raspberry Pi running a small script that forwards the data

- Option C: Bluetooth (HC-05) to a phone app

### 6. Backend / data layer

- Pick a platform: Firebase, Blynk, ThingSpeak, or a simple Flask/Node server

- Store the readings with timestamps

- Store the cleaning state (idle / awaiting permission / cleaning)

- Create an endpoint or database field for the permission response (approve/deny) that the Arduino can read

### 7. Dashboard

- Display turbidity, gas level and temperature

- Display the cleaning status (in progress or not)

- Show threshold indicators (e.g., green/red status per parameter)

- Auto-refresh every 5 minutes

- Add a history chart (optional but useful for the demo)

- Add Approve / Deny buttons for the cleaning request

### 8. Notification system

- Choose a channel: Telegram bot, email, push notification (Firebase), or in-dashboard alert

- Send a message like: "Turbidity and gas levels exceeded. Start cleaning? Yes / No"

- Capture the user's reply and pass it to the Arduino

- Add a timeout if there is no response, and decide the default (do nothing)

### 9. Integration and testing

- Simulate threshold crossings and confirm the notification arrives

- Confirm the pump starts only after permission is granted

- Confirm the pump stops at the correct level or on timeout

- Confirm the dashboard updates every 5 minutes and shows the correct status

- Test failure cases: Wi-Fi drop, sensor disconnected, no response to the notification

### 10. Documentation and demo

- Draw the circuit diagram and block diagram

- Save the final code and threshold table

- Prepare a flowchart of the logic

- Prepare the demo with dirty water vs. clean water